ICAM — Product Development Sprint

PlumeAI

AI + Satellite Methane Leak Detection

Full Project Research & Strategy Document

5-Day Presentation Sprint  ·  July 2026

Document Sections

01  The Problem — Scale & Context

02  Why Methane Matters — The Climate Science

03  Industries & Sources Affected

04  Current Detection Methods & Their Gaps

05  Economic & Regulatory Context

06  Existing Competitors in This Space

07  Our Problem Statement

08  The Detection Trilemma — The Real Gap

09  Our Solution — How PlumeAI Works

10  Team Name Options

# 01  The Problem — Scale & Context

The oil and gas industry is one of the largest single sources of human-caused methane emissions on Earth. Despite a decade of pledges and regulatory pressure, emissions have plateaued near record highs rather than declining.

## How big is the methane leak problem?

- The oil & gas sector leaks an estimated 80 million tonnes (Mt) of methane per year — roughly two-thirds the size of the entire global coal sector's methane footprint (IEA Global Methane Tracker, 2025–2026).

- Total global anthropogenic methane emissions are approximately 380–400 Mt/yr. Oil & gas represents around 20–22% of that total — making it the second-largest human sector after agriculture.

- Despite the Global Methane Pledge signed by 159 countries in 2021 (targeting a 30% cut by 2030), IEA data shows oil & gas methane emissions have not meaningfully declined.

- The IEA estimates that 30% of oil & gas methane emissions could be eliminated at zero net cost — the value of the captured gas pays for the fix. The barrier is not economics: it is detection.

## Key data points

| Metric | Figure |
| --- | --- |
| Annual O&G methane leaks | ~80 million tonnes/yr |
| Share of global anthropogenic CH₄ | ~20–22% |
| Fixable at zero net cost | ~30% of total (IEA) |
| Lost product value | ~$34 billion/yr (UNEP) |
| Countries signed Global Methane Pledge | 159 nations |

# 02  Why Methane Matters — The Climate Science

Methane (CH₄) is the second most important anthropogenic greenhouse gas after CO₂, but it is significantly more potent over shorter timescales — making it the fastest available lever to slow near-term warming.

## Global Warming Potential (GWP)

- Over a 20-year horizon (GWP-20): methane traps approximately 80× more heat per tonne than CO₂.

- Over a 100-year horizon (GWP-100): approximately 28–34× more potent than CO₂ (IPCC AR6, 2021).

- Unlike CO₂, which persists in the atmosphere for centuries, methane breaks down in roughly 12 years — meaning cuts today deliver measurable climate benefit within a decade.

## Contribution to historical warming

- Methane is responsible for approximately 30% of global warming since the pre-industrial era (IPCC AR6).

- The IEA estimates that eliminating all technically feasible oil & gas methane emissions would have the same near-term climate impact as closing all of the world's coal-fired power plants.

## Why oil & gas specifically?

- Agriculture (livestock and rice paddies) is the largest methane sector but is structurally harder to address — emissions are biogenic, diffuse, and tied to food supply.

- Coal methane is released during mining and hard to separate from ventilation systems.

- Oil & gas leaks are 'fugitive' — unintentional, point-source, and fixable with existing technology. This makes them the highest-leverage near-term target.

# 03  Industries & Sources Affected

## The oil & gas value chain

- Upstream (production): Well completions, pneumatic devices, storage tanks, and flaring. The largest source of fugitive emissions — responsible for ~60% of O&G sector leaks.

- Midstream (transport & pipelines): Compressor stations, valves, and pipeline infrastructure. A major source of 'super-emitter' events due to aging infrastructure.

- Downstream (refining & distribution): Refineries, distribution terminals, city gas networks. Smaller fraction but operationally significant.

- LNG (liquefied natural gas): Liquefaction, shipping, and regasification terminals — growing rapidly as a source due to LNG trade expansion.

## Super-emitter events

A small number of large events ('super-emitters') account for a disproportionate share of total emissions. Studies suggest that ~5% of facilities may account for 50–80% of total O&G methane emissions in a given region. Identifying and fixing super-emitters delivers outsized climate impact per dollar spent.

## Largest emitting countries / regions

| Country / Region | Key context |
| --- | --- |
| United States | Largest absolute O&G methane emitter; Permian Basin a major hotspot |
| Russia | Aging Soviet-era pipeline infrastructure; significant underreporting |
| Turkmenistan | Among highest per-GDP emitters; satellite data revealed systematic underreporting |
| Venezuela | Orinoco Belt; infrastructure degradation driving very high emission rates |
| Middle East / OPEC | High flaring rates; gas associated with oil production routinely vented |
| Kazakhstan | Major oil fields with limited monitoring infrastructure |

# 04  Current Detection Methods & Their Gaps

No single existing method delivers the combination of global coverage, facility-level precision, and low-cost/high-frequency revisit that is needed for effective monitoring at scale. This is the core gap that PlumeAI addresses.

## Detection method comparison

| Method | Coverage | Resolution | Speed / Cost | Key limitation |
| --- | --- | --- | --- | --- |
| TROPOMI / Sentinel-5P | Global, daily | ~7×5.5 km (too coarse) | Free, fast | Cannot attribute to a specific facility |
| GHGSat / Carbon Mapper | Targeted, tasked | 25–30 m (facility-level) | Must be scheduled; commercial cost | Narrow footprint; doesn't scale globally |
| OGI cameras / drones | Site-level only | Very high | Manual, slow, expensive | Cannot scale to thousands of sites |
| Self-reporting (factor-based) | All facilities | N/A — estimated | Annual reporting cycle | Systematically underestimates real emissions by 40–60%+ |
| MethaneSAT (EDF) | Regional | Intermediate | Lost contact; shut down 2025 | Proof that this is genuinely hard — hardware risk |
| PlumeAI (our approach) | Global, continuous | Facility-level via AI | <48 h; near-zero marginal cost | Uses free Sentinel-2 & Landsat imagery |

# 05  Economic & Regulatory Context

## Economic value of leaked methane

- UNEP estimates the leaked methane from oil & gas operations globally is worth approximately $34 billion per year in lost product value at current gas prices.

- This means a large share of the abatement cost is self-financing — operators who fix leaks recover a saleable product. The barrier is the cost and difficulty of detection, not the economics of repair.

- The MRV (Measurement, Reporting, and Verification) technology market for methane monitoring is projected to grow significantly — estimates range from $2B to $5B+ by 2030 as regulatory mandates take effect globally.

## Key regulatory drivers

- EU Methane Regulation (2024): Entered into force in 2024. Requires operators in the EU — and importers selling gas into the EU — to measure, report, and verify methane emissions at source. Non-compliant imports face market access restrictions by 2030, creating strong incentive for exporters in the US, Russia, and the Middle East to monitor and certify.

- US EPA OOOOb/c Rules (2024): Tightened requirements for leak detection and repair (LDAR) across US oil & gas operations, including super-emitter response programs triggered by satellite or aerial detection.

- OGMP 2.0 (Oil & Gas Methane Partnership): The gold-standard voluntary reporting framework from UNEP, requiring measurement-based (not factor-based) reporting at facility level. Over 80 major operators have joined, representing >30% of global oil & gas production.

- Global Methane Pledge (2021): 159 countries committed to a 30% reduction in global methane emissions by 2030 versus 2020 levels. Creates political pressure for monitoring infrastructure investment globally.

# 06  Existing Competitors in This Space

Several companies are already commercialising AI + satellite methane detection. Understanding this landscape helps us position PlumeAI's differentiation.

## Key players

- GHGSat (Canada): Pioneer in purpose-built methane-sensing nanosatellites. Very high resolution (25 m) but narrow footprint — must be tasked to specific sites. Premium commercial pricing limits accessibility for smaller operators and regulators in developing countries.

- Kayrros (France): Uses TROPOMI satellite data + AI to produce near-real-time global methane maps. Strong on regional/country-level monitoring but limited facility attribution due to TROPOMI's coarse resolution.

- Carbon Mapper: Partnership between Planet, NASA JPL, and US states. Uses hyperspectral aircraft and the EMIT satellite instrument for high-resolution point-source detection. Strong scientific credibility but limited commercial scalability.

- Orbio Earth: The closest proof point for our approach — uses free multispectral satellite imagery (Sentinel-2 SWIR) + AI to detect methane. A 2024 Stanford-led blind study found Orbio correctly identified every valid controlled methane release tested, while a hardware-based competitor missed one.

- MethaneSAT (EDF): Purpose-built satellite launched 2024, lost contact and shut down in 2025 after 15 months — a cautionary tale for hardware-only approaches.

## Our differentiation opportunity

- Use free, already-operational satellite data (Sentinel-2, Landsat) — no hardware risk, no satellite development cost.

- Target accessibility: small-to-mid operators, national regulators in emerging markets, and OGMP 2.0 compliance reporting — segments underserved by premium players.

- Open and auditable: position as a transparent, verifiable alternative to black-box commercial tools — important for regulatory credibility.

# 07  Our Problem Statement

"The oil and gas industry leaks an estimated 80 million tonnes of methane annually — roughly two-thirds the size of the entire global coal sector's methane footprint — and despite a decade of pledges, these emissions have plateaued near record highs rather than declined."

Methane is responsible for around 30% of global warming since the Industrial Revolution, and over a 20-year horizon it traps roughly 80 times more heat than CO₂. Because it breaks down in the atmosphere in ~12 years (vs. centuries for CO₂), cutting it is the fastest available lever to slow near-term warming.

The IEA estimates that 30% of oil & gas methane emissions could be eliminated at zero net cost — the leaked gas is worth enough to pay for the fix (~$34 billion/year in lost product value). The problem isn't that abatement is expensive. It's that most leaks are never found, or are found too late to act on.

## One-sentence version

"30% of oil & gas methane leaks could be fixed at zero net cost — the technology gap isn't the fix, it's finding the leak fast enough, cheaply enough, and precisely enough to act on it."

# 08  The Detection Trilemma — The Real Gap

The core issue is that no existing tool delivers coverage, resolution, and speed/cost simultaneously. Operators and regulators are stuck choosing between seeing everything blurrily or seeing one site sharply — never both, and rarely fast enough to stop a leak.

## The trilemma in detail

- Coverage vs. Resolution: TROPOMI covers the globe daily but at 7×5.5 km resolution — too coarse to say which facility is leaking, let alone which valve or compressor.

- Resolution vs. Cost: GHGSat resolves to 25 m (facility level) but must be 'tasked' to a specific location and costs are commercial — impractical for continuous monitoring of thousands of sites globally.

- Speed vs. Accuracy: Self-reported factor-based inventories cover all facilities annually but systematically underestimate real emissions by 40–60% or more. Canada's shift to measurement-based reporting raised its official methane numbers by over 35%.

## Why this is a software gap, not a hardware gap

The insight that drives our project: Sentinel-2 and Landsat are already up there, imaging every oil field on Earth every 5 days, for free. Their shortwave infrared (SWIR) bands are precisely the wavelengths where methane absorbs light — the signal is in the data. What's missing is the AI layer that reads it fast enough, at facility-level precision, to be actionable. This is what PlumeAI builds.

# 09  Our Solution — How PlumeAI Works

PlumeAI is an AI-powered pipeline that turns freely available satellite imagery into fast, precise, attributable methane leak detection — requiring no new hardware, no manual surveys, and no satellite tasking.

## The four-step pipeline

### Step 01 — Satellite data ingestion

- Data sources: Sentinel-2 (ESA, free, 10 m resolution, 5-day revisit) and Landsat-8/9 (USGS, free, 30 m, 16-day revisit).

- Key bands: Shortwave infrared (SWIR) bands 11 and 12, where methane absorbs solar radiation and leaves a detectable spectral signature.

- Coverage: Global. Every major oil & gas basin on Earth is imaged regularly at no marginal cost.

### Step 02 — AI spectral & spatial analysis

- A deep learning model (CNN or Vision Transformer) trained on known methane plume spectral signatures detects anomalies in the SWIR bands.

- Pre-processing pipeline removes cloud cover, atmospheric aerosols, and terrain reflectance artifacts.

- Multitemporal comparison: the model compares each new image against a baseline from the same site — reducing false positives from surface variation.

- Output: a probability heatmap of methane presence across the scene.

### Step 03 — Plume detection & source attribution

- Detected anomalies are cross-referenced with oil & gas facility location databases (Global Energy Monitor, national registries).

- Wind-field back-tracing (using ERA5 reanalysis meteorological data) traces plumes back to their source facility or equipment cluster.

- Emission rate estimation in kg/hr — the metric required by OGMP 2.0 and EU Methane Regulation.

- Super-emitter flagging: events exceeding regulatory thresholds are automatically escalated.

### Step 04 — Alert, dashboard & regulatory reporting

- Operators and regulators receive alerts within 24–48 hours of a satellite pass, with GPS coordinates, estimated emission rate, confidence score, and before/after imagery evidence.

- Dashboard provides historical trend view per facility — enabling operators to track repair effectiveness over time.

- Reports formatted for OGMP 2.0 and EU Methane Regulation disclosure requirements — reducing compliance burden.

## Why our approach bridges the gap

| Pillar | What we do | The advantage |
| --- | --- | --- |
| Zero data cost | Sentinel-2 & Landsat (free, global, daily) | No hardware investment, no satellite tasking fees |
| Speed | Automated AI pipeline processes imagery on ingestion | Detection in <48 h vs months of reporting lag |
| Precision | AI narrows km-scale blobs to individual facilities | Actionable alerts — which site, which area, how much |
| Scale | Runs on cloud compute, no marginal cost per site | Monitor 10,000 facilities as easily as 10 |
| Compliance-ready | Outputs formatted for OGMP 2.0, EU Methane Reg. | Reduces regulatory reporting burden for operators |

# 10  Team Name Options

Below are the team name suggestions discussed, grouped by angle. The top 3 recommendations are highlighted.

## Top 3 recommended names

- Flare Zero — Bold, memorable, signals the mission instantly. 'Zero' connects to zero-net-cost abatement and the goal of eliminating flaring.

- PlumeAI — Uses real industry terminology (a 'plume' is the gas cloud satellites detect). Clean, professional, immediately communicates what we do.

- GhostGas — Clever and memorable: methane is literally invisible — the 'ghost' pollutant nobody can see without technology.

## All options by category

### Satellite / Space angle

- OrbitalGuard

  - ClearSkies AI

    - ZenithWatch

      - SentinelAI (nod to Sentinel-2 satellite)

        - NadirTech (nadir = point directly below a satellite)

### Climate / Environmental angle

- ZeroLeak

  - NetherWatch (methane comes from below)

    - CleanFlare

      - AtmosGuard

        - PledgeAI (nod to the Global Methane Pledge)

### Smart / Technical angle

- LeakSense

  - PlumeAI

    - SpectraLeak

      - MethAI

        - CH₄Cast (CH₄ = methane's chemical formula)

### Bold / Startup-sounding

- Flare Zero

  - GhostGas

    - DarkPlume

      - SilentLeak

        - Fugitive AI (fugitive emissions = industry term for unintentional leaks)

ICAM Product Development Sprint  ·  PlumeAI  ·  July 2026  ·  Confidential — For Team Use Only